# Gaia-Argus Synthetic Testing.

This is a quick step by step guide for testing each module with synthetic signals.

Once each stage passes we will move onto testing with sensors.

### Phase 1: Data Intake and Core Visualization

1. Data Ingestion & Live Waveform

- Synthetic Input: Generate a 100 Hz, 3-component miniSEED file containing a pure 1 Hz sine wave with a known amplitude and a deliberate DC offset.

- Test Execution: Load the file via the offline miniSEED mode or the Omega Directory Watcher. Open the Live Waveform module.

- Expected Outcome:

  - The Canvas Waveform renderer must display the loaded trace immediately.

  - Applying the Box Select Mode and dragging across the sine wave should produce an Amplitude readout matching the synthetic input's known amplitude.

  - The chart should enforce the render cap of 4,000 points per visible channel.

- Actual Outcome:

2. QC Matrix

- Synthetic Input: A synthetic SeedLink stream where specific packets are intentionally dropped to create sequence discontinuities.

- Test Execution: Connect the stream and monitor the QC Matrix table. Toggle the Mute switch for the affected stream.

- Expected Outcome:

  - The dropped packets must register on the Gap counter, changing the display to a red badge showing the number of gap events.

  - Clicking the Mute toggle must change the status pill from Active to Muted (amber).

- Actual Outcome:

3. Helicorder

- Synthetic Input: A 24-hour continuous synthetic background noise file punctuated by distinct, high-amplitude impulse spikes at known intervals.

- Test Execution: Load the file and navigate to the Helicorder view.

- Expected Outcome:

  - The drum plot must show time advancing left to right within each row, wrapping to the next row below.

  - The display must lock every row to the same Y-axis scale based on the global peak amplitude.

  - The synthetic impulses must appear as sharp vertical deflections on the respective time rows.

- Actual Outcome:

### Phase 2: Spatial and Wavefield Analysis

4. Network Map & Polarization Analysis

- Synthetic Input: A 3-component trace simulating a perfect linear P-wave (identical, in-phase sine waves on Z, N, and E) yielding a theoretical 45^{\circ} back-azimuth and high rectilinearity. Ensure the synthetic PGA exceeds 0.02 g.

- Test Execution: Run the Polarization analysis with the rectilinearity threshold set to 0.7. Observe the Network Map.

- Expected Outcome:

  - The Polarization module must report a Rectilinearity (R) approaching 1.0 (perfect linear P-wave) and trigger the P-WAVE TRIGGER badge.

  - On the Network Map, the station marker must transition to an amber/red colour and pulse due to the > 0.02 g PGA.

  - The Network Map must automatically draw a polyline vector from the station marker in the computed back-azimuth direction.

- Actual Outcome:

5. Orbit 3D

- Synthetic Input: Use the same linear P-wave synthetic file from the Polarization test.

- Test Execution: Open the Orbit 3D module to visualize the 3-C displacement.

- Expected Outcome: The 3D scatter plot must render an elongated, needle-like trajectory aligned with the direction of propagation.

- Actual Outcome:

### Phase 3: Spectral and Structural Analysis

6. Response Spectra & Spectrogram

- Synthetic Input: A synthetic sum of two pure sine waves (e.g., 2 Hz and 10 Hz) followed by a swept-sine (chirp) signal.

- Test Execution: Capture a snapshot in the Response Spectra module. Compute the Spectrogram using NFFT=256.

- Expected Outcome:

  - The Response Spectra PSD plot must show sharp spectral peaks at exactly 2 Hz and 10 Hz.

  - The Spectrogram heatmap must plot Frequency on the Y-axis and Time on the X-axis, displaying the chirp as a diagonal line across the time-frequency matrix.

- Actual Outcome:

7. Kinematics

- Synthetic Input: A single sinusoidal acceleration pulse with a built-in DC offset.

- Test Execution: Observe the integrated velocity and displacement traces. Toggle the High-Pass (HP) filter.

- Expected Outcome:

  - Without the HP filter, the DC offset must integrate into a ramping velocity signal.

  - When the HP filter is enabled, the integration drift must be suppressed, returning the post-motion velocity baseline to flat.

- Actual Outcome:

8. OMA / FDD

- Synthetic Input: Two channels of synthetic broadband noise, both filtered to contain a hidden, shared resonance peak at exactly 2.5 Hz.

- Test Execution: Run the FDD analysis with the FFT block size (nperseg) set to 1024.

- Expected Outcome:

  - The SV1 curve (teal) on the FDD plot must show a distinct, isolated peak at 2.5 Hz.

  - The Modal Properties Table must automatically detect and list this natural frequency alongside its half-power damping ratio.

- Actual Outcome:

9. HVSR

- Synthetic Input: 3-component noise where the Z component has a spectral notch at 3 Hz, and the N/E components have corresponding peaks at 3 Hz.

- Test Execution: Assign the Z, N, and E streams and compute the HVSR.

- Expected Outcome: The H/V ratio spectrum (teal curve) must display a peak at the fundamental resonance frequency f_0 = 3 Hz.

- Actual Outcome:

10. Ambient Noise (\Delta v/v)

- Synthetic Input: Generate two identical noise traces for the reference stack. For the subsequent "current" block, artificially shift one trace by a known time delay (\delta t).

- Test Execution: Process the reference block, then compute \Delta v/v using the shifted block. Set an Alert Threshold of -1.0%.

- Expected Outcome:

  - The \Delta v/v Timeline must plot the calculated velocity change resulting from the time delay.

  - If the computed change is equal to or more negative than the -1.0% threshold, the red alert banner must appear at the top of the module.

- Actual Outcome:

### Phase 4: Event Refinement and Array Operations

11. Event Catalog, Phase Picker & HypoDD

- Synthetic Input: Inject two synthetic events into the SQLite historian with predefined P and S arrival times.

- Test Execution: Open the Phase Picker to drag the arrival markers. Select \ge2 events in the Event Catalog and click "Run HypoDD".

- Expected Outcome:

  - In the Phase Picker, dragging the pick lines must update the timestamp in real-time.

  - The HypoDD solver must complete and automatically open a 3D Plotly modal showing the original hypocenters (faded markers) and relocated positions (solid markers).

- Actual Outcome:

12. Site Characterization (MASW)

- Synthetic Input: A multi-channel synthetic surface wave record with known theoretical dispersion properties.

- Test Execution: Compute the MASW Image, click to pick the dispersion curve, and invert the picks.

- Expected Outcome:

  - The dispersion energy image must render as a frequency vs. phase-velocity heatmap.

  - The inversion must plot a layered-earth Vs depth profile as a step function.

  - The module must display the appropriate NEHRP site class badge based on the computed Vs30.

- Actual Outcome: